# Industrial Case Study

Prediction of brine-plume dispersion, evolution and seabed distribution for the Perth Seawater Desalination Plant submerged diffuser (Cockburn Sound, Western Australia)

Modelling tool: NEREID-B (solver.py, Rev 1.5) — 3-D finite-volume coupled brine dispersion solver with near-field correlation coupling, buoyancy-modified realizable k–ε turbulence, full anisotropic dispersion tensor and a Monte-Carlo stochastic ensemble.

# 1.  Executive summary

NEREID-B was applied to predict the fate of the hypersaline reject (brine) discharged from the Perth SWRO plant's submerged multiport diffuser into Cockburn Sound. The model resolves the near-field jet/plume via validated inclined-dense-jet correlations and the 3-D far-field gravity-current spreading and dilution of the negatively-buoyant plume over the seabed, under the site's ambient current, stratification and wave climate. A 4-member stochastic ensemble quantifies turbulence-driven uncertainty.

Headline predictions (steady, quasi-equilibrium plume):

Source: discharge salinity 61.4 g/kg into 36.5 g/kg ambient (excess at source 24.9 g/kg);

Near field: near-field (validated correlations): densimetric Froude number Fr=31.6, terminal rise 9.0 m, seabed return at 9.9 m, return dilution 29:1;

At 50 m: modelled brine dilution at the 50 m compliance boundary ≈ 47:1, excess salinity ΔS ≈ 0.53 g/kg;

Far field: peak salinity 37.56 g/kg, max excess 1.05 g/kg, seabed footprint above 1.2 g/kg ≈ 0 m², affected water volume ≈ 0 m³.

Mixing-zone assessment: the WA EPA criterion is ΔS < 1.2 g/kg within 50 m of the diffuser. Predicted ΔS at 50 m ≈ 0.53 g/kg → COMPLIANT. The model carries a documented CONSERVATIVE bias (under-predicts dilution ⇒ over-predicts impact), so the prediction is on the safe side.

# 2.  Site description and problem statement

The Perth Seawater Desalination Plant (Stage 1, ~45 GL/yr, 45% recovery) discharges its RO reject through a ~163 m double-tee submerged diffuser with 40 ports (0.13 m diameter) inclined at 60° in ~10 m of water in Cockburn Sound. The reject (~61.4 g/kg) is roughly 1.7× the ambient salinity (~36.5 g/kg) and therefore negatively buoyant: it rises briefly as a turbulent jet, falls back to the seabed, and spreads as a dense gravity current. The regulatory question is whether the diffuser dilutes the brine enough that the seabed excess-salinity footprint and the mixing-zone concentrations remain within ecological limits.

Prediction objectives for NEREID-B:

the near-field rise height, seabed return distance and return dilution;

the steady 3-D distribution of excess salinity and brine dilution over the seabed;

the centreline dilution and excess-salinity decay with distance from the diffuser;

the seabed footprint area exceeding the regulatory threshold and the affected volume;

the vertical structure of the plume (depth of the dense layer);

compliance with the mixing-zone criteria, with a stochastic uncertainty band.

# 3.  Input data (model deck)

All inputs are taken from the authoritative WA EPA App-D discharge-modelling validation report and standard Cockburn Sound met-ocean values. The full machine-readable deck is saved as 5/perth_case_input.json.

3.1  Discharge / diffuser

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Reject (brine) salinity | S₀ | 61.4 | g/kg |
| Reject temperature | T_b | 25.0 | °C |
| Flow per port | Q | 0.0628 | m³/s |
| Port diameter | d | 0.130 | m |
| Nozzle elevation angle | θ | 60 | deg |
| Number of ports | n | 40 | — |
| Port spacing | s | 4.1 | m |
| Nozzle height above bed | z₀ | 1.0 | m |

3.2  Ambient sea (receiving water)

| Parameter | Surface | Bottom | Unit |
| --- | --- | --- | --- |
| Ambient salinity | 36.5 | 36.7 | g/kg |
| Ambient temperature | 21.0 | 19.0 | °C |
| Water depth (diffuser) | 10.0 | — | m |

3.3  Met-ocean forcing

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Ambient current | U_c | 0.08 | m/s |
| Tidal current amplitude | U_tide | 0.03 | m/s |
| Significant wave height | H_s | 0.8 | m |
| Wave period | T_w | 6.0 | s |
| Wind speed (10 m) | U₁₀ | 6.0 | m/s |
| Latitude | φ | -32.2 | deg |

3.4  Numerical configuration

| Parameter | Value | Parameter | Value |
| --- | --- | --- | --- |
| Domain (Lx×Ly×depth) | 180×90×10 m | Grid (nx×ny×nz) | 36×22×14 |
| Simulated time | 280 s | Ensemble members | 4 |
| Near-field coupling | True | Stochastic forcing | True |
| Regulatory threshold ΔS_crit | 1.2 g/kg | Free surface | False |

# 4.  Methodology

NEREID-B solves the coupled system of §model.docx: incompressible RANS momentum with full nonlinear buoyancy, absolute-salinity and temperature transport with a full anisotropic dispersion tensor, a nonlinear equation of state, and a buoyancy-modified realizable k–ε turbulence closure, advanced by a fractional-step finite-volume scheme on a partial-cell bathymetry grid. The unresolvable sub-grid nozzle is represented by validated inclined-dense-jet correlations (Roberts et al. 1997) that seed the 3-D far field with the diluted return plume; the 3-D model then resolves the seabed gravity-current spreading and mixing. A Monte-Carlo ensemble of stochastically-forced realisations yields the mean field and an exceedance-probability map. The model is validated to be conservative across the published Perth multi-point transect and the universal Roberts (1997) dense-jet scaling.

# 5.  Predicted results

5.1  Headline metrics

| Quantity | Predicted value | Unit |
| --- | --- | --- |
| Densimetric Froude number, Fr_d | 31.6 | — |
| Near-field terminal rise height | 9.0 | m |
| Near-field seabed return distance | 9.9 | m |
| Near-field return dilution | 29 | :1 |
| Peak salinity (S_max) | 37.56 | g/kg |
| Max excess salinity (ΔS_max) | 1.05 | g/kg |
| Brine dilution at 50 m | 47 | :1 |
| Excess salinity ΔS at 50 m | 0.53 | g/kg |
| Excess salinity ΔS at 100 m | 0.28 | g/kg |
| Horizontal reach (r_max, >ΔS_crit) | 0.0 | m |
| Seabed footprint (>1.2 g/kg) | 0 | m² |
| Affected water volume | 0 | m³ |
| Dense-layer deepest impact | 0.0 | m below surface |

5.3  Stochastic uncertainty (ensemble)

Across the 4-member ensemble the peak-excess standard deviation is 0.256 g/kg and the 95th-percentile excess reaches 1.05 g/kg, giving the exceedance-probability field below.

# 6.  Output figures, curves and charts

Figure 1. Predicted seabed excess-salinity map (plan view): the dense brine plume spreading from the diffuser, with the regulatory threshold contour.

Figure 2. Vertical section of excess salinity along the plume centreline, showing the bottom-trapped dense gravity-current layer.

Figure 3. Centreline brine dilution vs distance from the diffuser (Tier-5 curve: curve_centerline.csv).

Figure 4. Excess-salinity ΔS decay with distance from the diffuser.

Figure 5. Exceedance-probability map for the regulatory threshold across the stochastic ensemble.

Figure 6. Near-bed current/velocity field driving the gravity-current spreading.

Figure 7. Near-field inclined dense-jet trajectory (validated correlation model).

# 7.  Output data files (folder 5/)

Every artifact below is generated by the solver run and saved in folder 5/:

| File | Type | Contents |
| --- | --- | --- |
| metrics_summary.json | JSON | headline metrics + full config + ensemble stats + active physics |
| metrics_timeseries.csv | CSV | time series: S_max, ΔS_max, reach, footprint, dilution, divergence |
| curve_centerline.csv | CSV | centreline curve: distance, ΔS, dilution, core depth |
| curve_vertical_profile.csv | CSV | vertical profile: depth, salinity, ΔS, density, temperature |
| fields_final.npz | binary | full 3-D fields: S, ΔS, dilution, ρ, u, v, w, T, k, ε, ν_t |
| ensemble_stats.npz | binary | ensemble mean / std / exceedance fields |
| fig_*.png | graphics | maps, sections, curves, risk/exceedance charts |
| run.log | log | solver health: divergence, mass balance, cap fractions, near-field summary |
| perth_case_input.json | JSON | the complete input deck for this case |

# 8.  Compliance assessment and conclusions

The predicted excess salinity at the 50 m mixing-zone boundary is ≈ 0.53 g/kg (dilution ≈ 47:1), to be compared with the WA EPA criterion of ΔS < 1.2 g/kg at 50 m. On this basis the discharge is assessed COMPLIANT. Because NEREID-B is validated to UNDER-predict dilution (and therefore OVER-predict salinity/impact) against both the published Perth transect and the universal Roberts (1997) dense-jet scaling, the prediction is conservative — the true field dilution is expected to be somewhat higher (lower impact).

Run health: divergence 0.0 (machine precision), mass imbalance 0.0, eddy-viscosity cap engaged in 0.0% of cells (physical, no railing).

Recommendations: (i) the conservative prediction supports the diffuser design against the 50 m criterion; (ii) a site CTD/ADCP transect would tighten the absolute far-field numbers; (iii) seasonal stratification and low-current ('worst-case' weak-mixing) scenarios should be run as additional ensemble cases to bound the compliance envelope.

# 9.  References & provenance

• WA EPA, 'Perth Desalination Plant Discharge Modelling: Model Validation' (App. D, PSDP referral).

• Roberts, P.J.W., Ferrier, A. & Daviero, G.J. (1997), 'Mixing in Inclined Dense Jets', J. Hydraul. Eng. 123(8):693–699.

• Roberts, P.J.W., Taplin, J. & Zigas, E. (2019), 'Design of Seawater Desalination Brine Diffusers', 38th IAHR World Congress, doi:10.3850/38WC092019-1053.

• Governing equations: 5/model.docx. Solver: solver.py (NEREID-B Rev 1.5). Input deck: 5/perth_case_input.json.