# NEREID-B — Governing Coupled PDE System

Nonlinear Eulerian Reactive-osmotic Effluent Integro-Dispersion model for a negatively-buoyant brine plume from an inclined submarine diffuser in a moving, stratified, wave-/tide-/wind-forced sea.

This document states the coupled system of partial differential equations that the reference solver (solver.py, Rev 1.5) discretises by a fractional-step finite-volume method. z is positive up with the still surface at z = 0; u = (u, v, w); bold symbols are vectors/tensors; repeated indices are summed.

# 1.  State vector and operators

The model advances the coupled state vector

q = ( u, v, w, p, S, T, ρ, k, ε, ζ, η )

over a fluid domain Ω(x, y, z, t) bounded below by the bathymetry z = −H(x, y) and above by the free surface z = η(x, y, t). The material derivative and the resolved (eddy-viscous) RANS closure use

D/Dt = ∂/∂t + (u · ∇) ,    Sᵢⱼ = ½ ( ∂uᵢ/∂xⱼ + ∂uⱼ/∂xᵢ ) ,    |S|² = 2 Sᵢⱼ Sᵢⱼ		(1)

# 2.  Continuity (mass conservation)

The flow is treated as incompressible (Boussinesq); the divergence-free constraint is enforced each step by the pressure projection:

∇ · u = 0		(2)

H5 (optional, non-Boussinesq / low-Mach): the projection instead solves the variable-density pressure-Poisson equation, so the local density enters the inertia and pressure coupling exactly:

∇ · ( (1/ρ) ∇p′ ) = (1/Δt) ∇ · u* ,    uⁿ⁺¹ = u* − Δt (1/ρ) ∇p′		(3)

# 3.  Momentum (buoyancy-modified RANS)

The Reynolds-averaged momentum balance with eddy-viscosity closure, full nonlinear buoyancy, rotation, and the resolved forcing terms:

∂uᵢ/∂t + ∂(uⱼ uᵢ)/∂xⱼ = − (1/ρ₀) ∂p/∂xᵢ + ∂/∂xⱼ[ (ν + ν_t)(∂uᵢ/∂xⱼ) ] + bᵢ − εᵢⱼ₃ f uⱼ + Fᵢ + ζᵢ		(4)

with the buoyancy (acting on the vertical component only)

bᵢ = − [ g (ρ − ρₐ) / ρ₀ ] δᵢ₃        (Boussinesq);     bᵢ = − [ g (ρ − ρₐ) / ρ ] δᵢ₃   (non-Boussinesq, H5)		(5)

where ρₐ(z) is the ambient (background) density, f = 2Ω sin φ the Coriolis parameter (beta-plane option f = f₀ + β(y − y₀), G11), and Fᵢ collects the body forces:

wind stress on the surface layer, τ = ρ_air C_d |U₁₀| U₁₀ ;

quadratic bottom drag on the bed cell, τ_b = ρ₀ C_d^bed |u_b| u_b (optional B1);

optional osmotic body force  F_os = − γ_os g ∇(S / S₀) ;

radiation-stress (wave-setup) force F_rad (H4, eq. 18);

and ζᵢ the stochastic acceleration (eq. 15). The pressure p is the Lagrange multiplier enforcing eq. (2); momentum advection is the divergence-consistent MAC flux, so no spurious φ(∇·u) is produced.

# 4.  Absolute-salinity transport

Salinity is advected by the divergence-free velocity and dispersed by the full anisotropic, state-dependent dispersion tensor D (eq. 9), with the thermodiffusive (Soret) and osmotic/reverse-osmosis cross-fluxes:

∂S/∂t + ∇ · (u S) = ∇ · ( D ∇S ) + σ_So ∇ · ( w_S D ∇T ) + ∇ · ( D_os ∇S ) + Q_S		(6)

The Soret salt flux is concentration-weighted (G8, conservative form) so it vanishes at zero and at saturation, and the osmotic effective diffusivity scales with the local salinity (because Π ∝ S ⇒ ∇Π ∝ ∇S):

w_S = 4 (S/S₀)(1 − S/S₀) ,        D_os = κ_os ( S / S₀ )		(7)

Q_S is the diluted nozzle source (near-field coupling, §11). The discrete scheme is monotone (TVD MUSCL) and mass-conserving, with a hard, mass-redistributing bound 0 ≤ S ≤ S₀ (G5).

# 5.  Temperature transport (with Dufour cross-flux)

Heat is transported analogously, with the reciprocal (Dufour) cross-flux driven by the salinity gradient:

∂T/∂t + ∇ · (u T) = ∇ · ( D_T ∇T ) + σ_So d_f ∇ · ( D_T ∇S ) + Q_T		(8)

where D_T = κ_T + ν_t / Pr_t (+ geometric anisotropy) and d_f is the explicit Dufour coefficient (named parameter, G8).

# 6.  Nonlinear equation of state (density coupling)

Density closes the buoyancy coupling. The default is a linear-contraction law with a quadratic cabbeling term:

ρ = ρ₀ [ 1 − α_T (T − T₀) + β_S (S − S₀) ] − ½ ρ₀ c_cab (T − T₀)²		(9)

H2 (optional, full_nonlinear, TEOS-10-style) adds higher-order T–S curvature and the pressure (thermobaric) dependence via the local hydrostatic pressure P:

ρ_full = ρ + ρ₀ [ ½ β_S2 (S−S₀)² − λ_TS (T−T₀)(S−S₀) + κ_p P − γ_tb P (T−T₀) ] ,   P = ρ₀ g |z| / 10⁴		(10)

# 7.  Anisotropic dispersion tensor

The symmetric, positive-definite dispersion tensor superposes an isotropic (molecular + turbulent) part, a flow-aligned Taylor-shear part, a wave-orbital part, and a bed-confined along-slope part:

D = ( D_m + ν_t / Sc_t + D_h ) I + D_sh ( ê_u ⊗ ê_u ) + D_w ( ê_w ⊗ ê_w ) + D_b ( I − n̂ ⊗ n̂ )		(11)

with ê_u the flow direction, ê_w the wave direction, n̂ the seabed normal, D_sh = a_sh |u_h| Δx (Taylor shear), D_w = g_w (π H_s / T_w) H_s (wave orbital), and D_b the bathymetric enhancement decaying with height above the bed. Each added term is positive semi-definite, so D stays SPD (well-posed anisotropic diffusion).

# 8.  Turbulence closure (buoyancy-modified, realizable k–ε)

Two transport equations close the eddy viscosity:

∂k/∂t + ∇·(u k) = ∇·[ (ν + ν_t/σ_k) ∇k ] + P_k + G_b − ε		(12)

∂ε/∂t + ∇·(u ε) = ∇·[ (ν + ν_t/σ_ε) ∇ε ] + (ε/k) [ C₁ ( P_k + C₃ max(G_b,0) ) − C₂ ε ]		(13)

with shear production P_k and the buoyancy production with its CORRECTED sign (damping for stable stratification, ∂ρ/∂z < 0 ⇒ G_b < 0):

P_k = ν_t |S|² ,        G_b = ( ν_t / Pr_t )( g / ρ₀ ) ∂ρ/∂z		(14)

The eddy viscosity uses the realizable (Durbin) time-scale limiter plus a Smagorinsky LES dissipation floor, so it cannot over-produce/rail on fine grids:

ν_t = C_μ k 𝒯 ,   𝒯 = min( k/ε , C_r / ( √6 C_μ √(|S|²) ) ) ,   ν_t ← max( ν_t , (C_s Δ)² √(|S|²) )		(15)

# 9.  Stochastic forcing (colored noise)

Each velocity channel is forced by a spatially-correlated Ornstein–Uhlenbeck process (stationary colored random stress), entering the momentum balance (eq. 3) as an acceleration:

∂ζᵢ/∂t = − (1/τ) ζᵢ + σ √(2/τ) · 𝒩ᵢ(x, t)		(16)

where 𝒩ᵢ is unit-variance noise smoothed to a spatial correlation length ℓ; τ is the de-correlation time and σ the r.m.s. intensity. A Monte-Carlo ensemble over realisations yields the mean, variance and exceedance probability of the salinity field.

# 10.  Free surface (kinematic + dynamic)

The rigid lid is replaced by an evolving free surface η(x, y, t) with the kinematic and (linearised) dynamic conditions, folded implicitly into the pressure solve (unconditionally stable):

∂η/∂t = w |_{z=η} ,        p |_{z=η} = ρ g η		(17)

# 11.  Near-field coupling (sub-grid nozzle boundary condition)

The unresolvable nozzle is represented by the validated empirical inclined-dense-jet correlations (Roberts et al. 1997; CORMIX/VISJET-class), which set the diluted seabed return state that seeds the 3-D far field. For the densimetric Froude number Fr = u_d / √(g′₀ d) of a 60° jet:

S_return = S_amb + (S₀ − S_amb) / (1.6 Fr) ,   x_return = 2.4 d Fr ,   z_rise = 2.2 d Fr		(18)

with reduced gravity g′₀ = g (ρ_amb − ρ_brine) / ρ_amb. This is the source term Q_S in eq. (4) and the corresponding momentum/temperature seeds.

# 12.  Surface-wave momentum coupling (optional, H4)

Monochromatic surface waves contribute a depth-decaying Stokes drift that advects all fields (Craik–Leibovich), and a radiation-stress gradient body force from shoaling:

u_s(z) = u_s0 e^{2kz} ê_w ,   u_s0 = g_st ω k a² ,   a = H_s/2 ,   ω = 2π/T_w ,   k = ω²/g		(19)

Transport velocity:  u_L = u + u_s ;     F_rad = − (1 / (ρ₀ H)) ∇ · S_xx ,   S_xx = E (2n − ½) ,   E = ½ ρ₀ g a²		(20)

# 13.  Initial, boundary, and closure conditions

Inflow (−x): prescribed ambient current U_in(t) = U_c + U_tide sin(2πt/T_M2), carrying ambient S, T.

Outflow (+x): open zero-gradient, or Sommerfeld/Orlanski radiation ∂φ/∂t + c ∂φ/∂x = 0 (H6).

Lateral (±y): free-slip, no-flux walls (optional sponge).

Seabed (z = −H): no-flux scalars; free-slip or log-law wall function + quadratic drag (optional).

Surface (z = η): wind stress, free-surface kinematics (§10), no scalar flux.

Ambient profiles: linear S_amb(z), T_amb(z) between bed and surface values; ρ_amb from eq. (6).

# 14.  Time integration

The default is a first-order fractional step (advection + cross-fluxes explicit; diagonal dispersion / turbulent stress backward-Euler implicit; MAC projection). The optional SSP-RK2 (Heun) scheme (H1) raises the temporal order to second:

q⁽¹⁾ = Φ(qⁿ) ,   q⁽²⁾ = Φ(q⁽¹⁾) ,   qⁿ⁺¹ = ½ ( qⁿ + q⁽²⁾ )		(21)

where Φ is one fractional-step update; the average of two divergence-free states is divergence-free.

# 15.  Symbol table

| Symbol | Meaning | Symbol | Meaning |
| --- | --- | --- | --- |
| u = (u,v,w) | velocity (m/s) | p | dynamic pressure (Pa) |
| S | absolute salinity (g/kg) | T | temperature (°C) |
| ρ, ρ₀, ρₐ | density / reference / ambient | η | free-surface elevation (m) |
| ν, ν_t | molecular / eddy viscosity | D, D_T | salt / heat dispersion tensor |
| k, ε | TKE / dissipation rate | P_k, G_b | shear / buoyancy production |
| C_μ,C₁,C₂,C₃ | k–ε constants | Sc_t, Pr_t | turbulent Schmidt / Prandtl no. |
| α_T, β_S | thermal exp. / haline contr. | c_cab | cabbeling coefficient |
| β_S2, λ_TS | haline / thermohaline curvature | κ_p, γ_tb | compressibility / thermobaric |
| σ_So, d_f | Soret / Dufour coefficients | κ_os | osmotic diffusivity |
| ζ, τ, σ, ℓ | OU forcing, time, intensity, scale | f, Ω, β | Coriolis / rotation / beta |
| u_s, k, a | Stokes drift / wavenumber / amp. | S_xx, E | radiation stress / wave energy |
| Fr, g′₀, d | Froude no. / reduced g / port dia. | Δ, C_s, C_r | grid scale / Smagorinsky / realiz. |

Reference implementation: solver.py (NEREID-B Rev 1.5). These equations are the continuous model; the solver is their numerical realisation (3-D structured finite-volume, partial-cell bathymetry, fractional-step Chorin projection, TVD-MUSCL advection, implicit anisotropic dispersion, LU-factorised variable-coefficient pressure-Poisson).