NEREID-B — Input Data Requirements

Data Acquisition, Measurement and Pre-Processing Plan Required to Drive the Coupled Stochastic PDE Brine-Salinity Model

Companion to salinity.docx & output.docx  ·  Rev. 1.0  ·  11 June 2026

This document specifies every input the NEREID-B model requires, how each input is obtained (measurement instrument, survey, model or literature), the spatial and temporal resolution and accuracy needed, and the update frequency. Inputs are grouped by physical category. Each table lists the symbol, the quantity, how it is acquired, the required resolution / accuracy, and its update cadence. A priority tier (Required / Recommended / Optional) is given so a project can scope a minimum-viable dataset and grow it. The document closes with the field-survey / monitoring programme, data-format and QA-QC requirements, and a readiness checklist.

Priority key:  R = Required (model will not run without it)   ·   S = Strongly recommended (materially affects accuracy)   ·   O = Optional (refinement / specific scenarios).

# 1. Input Data Classes

NEREID-B ingests seven classes of input:

A. Effluent / brine discharge characteristics (the source term).

B. Pipe & diffuser geometry and hydraulics (the boundary device).

C. Ambient sea state — physical (salinity, temperature, density, pressure).

D. Ambient sea dynamics — currents, tides, waves, turbulence (the moving sea).

E. Atmospheric / air–sea forcing (wind, heat, evaporation).

F. Bathymetry, terrain & domain geometry.

G. Thermodynamic, coupling & stochastic coefficients (closure data).

Classes A–B define the source and how it is injected; C–F define the receiving environment and its forcing; G supplies the physical constants and the statistics of the unpredictable forcing.

# 2. Class A — Effluent / Brine Discharge (Source Term)

| Sym. | Quantity | How acquired | Resolution / accuracy | Update | Pri. |
| --- | --- | --- | --- | --- | --- |
| S₀ | Brine salinity / TDS | In-line conductivity & density meter on reject line; lab TDS | ±0.1 g kg⁻¹; continuous | 1 min–1 h | R |
| Q_d | Discharge flow rate | Magnetic / ultrasonic flowmeter on outfall | ±2 % | 1 min | R |
| U_d | Nozzle exit velocity | Derived: Q_d / port area | from Q_d & geometry | 1 min | R |
| p_d | Discharge pressure | Pressure transducer at manifold/port | ±1 % | 1 min | S |
| T_b | Brine temperature | In-line RTD/thermistor | ±0.1 °C | 1 min | R |
| ρ_b | Brine density | Density meter or from S₀,T_b via EOS | ±0.5 kg m⁻³ | 1 min | R |
| composition | Ion composition (Na,Cl,Ca,Mg,SO₄) | Periodic lab ion chromatography | — | weekly | S |
| S_anti | Anti-scalant/additive load | Plant dosing records | as dosed | per batch | O |
| pH, DO | Reject pH, dissolved oxygen | In-line probes | ±0.1 pH | 1 h | O |

# 3. Class B — Pipe & Diffuser Geometry and Hydraulics

| Sym. | Quantity | How acquired | Resolution / accuracy | Update | Pri. |
| --- | --- | --- | --- | --- | --- |
| L_p | Pipe length | Engineering drawings / as-built survey | ±0.1 m | static | R |
| d_p | Pipe & port diameter | As-built / design spec | ±1 mm | static | R |
| ε_r | Wall roughness | Material spec; ROV inspection for ageing | — | annual | S |
| N_port, s_port | Port count & spacing | Diffuser design | exact | static | R |
| θ, ψ | Nozzle elevation & azimuth angle | Design + as-built ROV survey | ±1° | static | R |
| h_n | Nozzle height above bed | ROV / multibeam survey | ±0.1 m | annual | S |
| Δz_p | Pipe vertical rise | As-built profile | ±0.1 m | static | S |
| K_minor, f | Minor-loss & friction factors | Hydraulic design / handbook + calibration | — | static | S |
| x_d | Diffuser location (geo-position) | DGPS / survey | ±0.5 m | static | R |

# 4. Class C — Ambient Sea: Physical State

| Sym. | Quantity | How acquired | Resolution / accuracy | Update | Pri. |
| --- | --- | --- | --- | --- | --- |
| S_amb(z) | Ambient salinity profile | CTD casts; moored conductivity sensors | ±0.01 g kg⁻¹; ΔZ≈0.5 m | seasonal + real-time | R |
| T_amb(z) | Ambient temperature profile | CTD; thermistor chain | ±0.01 °C; ΔZ≈0.5 m | seasonal + real-time | R |
| ρ_amb(z) | Density / stratification | Derived from S,T,p via TEOS-10 | from CTD | seasonal | R |
| p(z) | Pressure profile | CTD pressure sensor / hydrostatic | ±0.1 dbar | static law | R |
| N²(z) | Stratification frequency | Computed from ρ(z) | from CTD | seasonal | S |
| c_p, μ | Specific heat, viscosity | Seawater property libs (from S,T,p) | literature | static | S |
| D_mol | Molecular salt diffusivity | Literature value (≈1.5×10⁻⁹ m² s⁻¹) | literature | static | S |
| turbidity/SS | Suspended-solid background | Optical turbidity sensor | ±5 % | monthly | O |

Profiles should span at least one full seasonal cycle to capture summer/winter stratification extremes, plus event-driven casts. A vertical resolution of ≈0.5 m near the pycnocline is recommended.

# 5. Class D — Ambient Sea Dynamics (the Moving Sea)

| Sym. | Quantity | How acquired | Resolution / accuracy | Update | Pri. |
| --- | --- | --- | --- | --- | --- |
| U_current(z,t) | Current speed & direction profile | ADCP (bottom-mounted/moored); HF radar; ocean model | ±1 cm s⁻¹; ΔZ≈1 m | 10 min, ≥30 d | R |
| A_j,ω_j,φ_j | Tidal harmonic constituents | Harmonic analysis of tide-gauge / ADCP record | ≥35 d record | per analysis | R |
| H_s,T_w,ê_w | Wave height, period, direction | Wave buoy / AWAC; wave model (SWAN) | ±10 % | 30 min | S |
| u_St | Stokes drift | Derived from wave spectrum | from waves | 30 min | O |
| spectra | Directional wave spectra | Directional wave buoy | — | 1 h | O |
| ω | Background vorticity/eddies | ADCP array / model | — | 10 min | O |
| φ_lat | Latitude (Coriolis f) | Site coordinates | exact | static | R |
| τ_m,σ_m,ℓ_m | Stochastic forcing statistics | Estimated from ADCP/wind time-series (autocorrelation, variance, length-scale) | from records | per calibration | S |

The stochastic-layer parameters (τ_m, σ_m, ℓ_m) are estimated by fitting Ornstein–Uhlenbeck statistics — de-correlation time, variance and spatial correlation length — to the observed current, wave and wind fluctuation records. These convert the measured variability of the sea into the model's uncertainty forcing.

# 6. Class E — Atmospheric / Air–Sea Forcing

| Sym. | Quantity | How acquired | Resolution / accuracy | Update | Pri. |
| --- | --- | --- | --- | --- | --- |
| U₁₀ | Wind speed & direction at 10 m | Met station / anemometer; reanalysis (ERA5) | ±0.5 m s⁻¹ | 10 min–1 h | S |
| σ_wind | Wind gust statistics | From anemometer record | — | per calibration | O |
| C_d | Air–sea drag coefficient | Parameterisation (wind-speed dependent) | literature | static | S |
| ρ_a, T_air | Air density & temperature | Met station / reanalysis | ±0.3 °C | 1 h | S |
| E, P | Evaporation & precipitation | Met station; flux parameterisation | ±10 % | daily | O |
| Q_rad | Net radiative heat flux | Pyranometer / reanalysis | ±10 W m⁻² | 1 h | O |
| RH | Relative humidity | Met station | ±3 % | 1 h | O |

# 7. Class F — Bathymetry, Terrain & Domain

| Sym. | Quantity | How acquired | Resolution / accuracy | Update | Pri. |
| --- | --- | --- | --- | --- | --- |
| H(x,y) | Bathymetry / seabed depth | Multibeam echosounder survey; LiDAR nearshore; ENC charts | ≤5 m grid; ±0.1 m | project + post-storm | R |
| ∇H | Bed slope / shelf geometry | Derived from H gradient | from DEM | static | R |
| seabed type | Sediment / substrate class | Grab samples; side-scan sonar | class map | project | S |
| C_D^bed | Bed friction coefficient | From substrate class / calibration | literature | static | S |
| coastline | Shoreline & structures | Survey / satellite / charts | ≤5 m | annual | R |
| L_x,L_y,D | Domain extent & depth | Modeller choice from site | — | static | R |
| receptors | Sensitive-habitat locations | Ecological survey (seagrass, reef) | GPS points | project | S |

# 8. Class G — Thermodynamic, Coupling & Closure Coefficients

| Sym. | Quantity | How acquired | Resolution / accuracy | Update | Pri. |
| --- | --- | --- | --- | --- | --- |
| α_T,β_S,γ_p | Expansion/contraction/compress. coeffs | TEOS-10 equation of state (from S,T,p) | standard | computed | R |
| φ_os | Osmotic (activity) coefficient | Pitzer model / lab osmometry for the brine | ±2 % | per brine | S |
| L_p | Osmotic permeability coefficient | Calibration / irreversible-thermo estimate | calibrated | per calibration | O |
| D_ST,D_TS | Soret & Dufour coefficients | Literature / lab; small for seawater | literature | static | O |
| Sc_t,Pr_t | Turbulent Schmidt & Prandtl numbers | Calibration (≈0.7) | calibrated | per calibration | S |
| C_μ,C₁,C₂,σ | k–ε turbulence constants | Standard values; calibration | standard | static | S |
| v̄_w,M_s,ν | Molar volume, salt molar mass, ion number | Chemistry of the brine | literature | static | S |

# 9. Field-Survey and Monitoring Programme

A staged data-acquisition campaign supplies the time-varying inputs and the calibration/validation data:

Baseline (pre-operation) survey: seasonal CTD grid, multibeam bathymetry, sediment sampling, habitat mapping — establishes ambient S,T,ρ climatology and the domain.

Mooring deployment: bottom-mounted ADCP + thermistor/conductivity chain + wave gauge for ≥30–45 days per season — supplies currents, tides, waves, stratification and the stochastic statistics.

Meteorological feed: on-site met station and/or ERA5 reanalysis — wind, heat, evaporation.

Source monitoring: continuous in-line instrumentation on the reject line (flow, conductivity, density, temperature, pressure).

Validation survey: CTD transects and towed/AUV salinity mapping around the operating outfall — the data the model is tested against (see output.docx, Tier 8b).

Event & post-storm surveys: re-survey bathymetry and re-sample currents after major storms to update H and forcing statistics.

# 10. Data Format, Pre-Processing and QA/QC

Coordinate reference: all spatial data in a single projected CRS (e.g. UTM) with a defined vertical datum (chart datum / MSL).

Time base: all time series in UTC with consistent sampling; gaps flagged, not zero-filled.

Formats: gridded fields as NetCDF/GeoTIFF; time series as CSV/Parquet; bathymetry as XYZ/DEM; metadata per CF conventions.

Pre-processing: de-spiking, drift correction and calibration of sensors; harmonic analysis to separate tidal/residual currents; interpolation of CTD casts to model levels.

QA/QC: range and spike checks, sensor cross-comparison, mass-balance sanity checks, and documented uncertainty for every input (feeds the stochastic layer and the output confidence envelopes).

Gap-filling: short gaps by interpolation; long gaps by validated ocean/atmospheric model reanalysis, with the substitution flagged.

# 11. Minimum-Viable vs Full Dataset

The model can run at increasing fidelity as data is added:

| Tier | Dataset | Enables |
| --- | --- | --- |
| Minimum (R only) | Discharge S₀/Q_d/T_b, pipe geometry & angle, ambient S/T profile, bathymetry, currents+tides, latitude | Deterministic unsteady plume; r_max, z_max, dilution, footprint |
| Recommended (R+S) | Adds waves, wind, stratification time series, osmotic & turbulence coefficients, stochastic statistics | Full coupled accuracy + uncertainty envelopes & exceedance maps |
| Full (R+S+O) | Adds ion chemistry, heat fluxes, Soret/Dufour, directional spectra, event surveys | Maximum-fidelity research-grade prediction & all novel couplings |

# 12. Input-Readiness Checklist

Before a production run, confirm:

☐ Continuous source monitoring (flow, salinity, density, T, p) live.

☐ As-built diffuser geometry, port angles and position verified by ROV.

☐ Seasonal CTD profiles of ambient S, T, ρ available.

☐ ≥30-day ADCP current record + tidal harmonics derived.

☐ Wave and wind time series (measured or reanalysis) secured.

☐ Multibeam bathymetry / DEM on a consistent CRS & datum.

☐ Osmotic, turbulence and stochastic coefficients estimated/calibrated.

☐ Validation dataset (outfall CTD survey) reserved for model testing.

☐ All inputs QA/QC'd with documented uncertainties.

Note on inputs easily overlooked but required here: the stochastic-forcing statistics (τ_m, σ_m, ℓ_m) that quantify the sea's unpredictability, the osmotic/activity coefficient of the specific brine, sensor uncertainties (which propagate into the output confidence envelopes), the vertical datum reconciliation between bathymetry and tide data, and post-storm bathymetry refresh — each is included above because NEREID-B's coupled, stochastic formulation depends on them.